CHAPTER 1

INTRODUCTION

The  16 channels relay output board  for pocket 16 DO provides relay output functions, which  provides 16 Single Pole  Single  Throw (SPST)  relays to drive 16  different  output channels.  Each relay channel can be used  to control  ON  / OFF of  external  devices,  to drive external high power relays, to activate alarms ... etc.

The features of 16 channels relay output board are:

- Support 16 relay output channels.

- Max  contact rating for relay:  120V  AC/DC 1AMP.

- Attraction time for relay: 3 ms.

- Fall off time for relay: 2 ms.

- Isolation resistance for relay: 100M OHM.

- Life  expectancy   for  relay:  5   million  operation at full load.

- The Normal Open (NO)  and  Common  contacts   (COM)  of each relay is brought out to  the  screw connector.

- LED  indicates when corresponds channel  is  activated.

- Operating temperature 0 to 60 ºC.

- Storage temperature –20 to 70 degree ºC.

- Humidity 5% to 95% in non-condensing.

1. Terminal Board

The  figure  of pocket  16  DO  terminal board is shown on the following page.

CHAPTER 2

HARDWARE CONFIGURATION

Before you use the  terminal board,  you must be sure to have this board plugged  into pocket 16 DO box.

The  LED17 in terminal board  shows  the power  on status. When it turns to light,  we have  power to this board, otherwise when  it does not light, no power is provided.

2.2 Pin Assignment of T1

| Pin | Function |  | Pin |  | Function |
| --- | --- | --- | --- | --- | --- |
| 1 | GND |  | 14 |  | +5V |
| 2 | DO10 |  | 15 |  | GND |
| 3 | DO11 |  | 16 |  | GND |
| 4 | DO12 |  | 17 |  | GND |
| 5 | DO13 |  | 18 |  | GND |
| 6 | DO14 |  | 19 |  | GND |
| 7 | DO15 |  | 20 |  | DO23 |
| 8 | DO16 |  | 21 |  | DO24 |
| 9 | DO17 |  | 22 |  | DO25 |
| 10 | DO20 |  | 23 |  | DO26 |
| 11 | DO21 |  | 24 |  | DO27 |
| 12 | DO22 |  | 25 |  | +5V |
| 13 |  | GND |  |  |  |

2.3  Pin Assignment of TB1

| Pin | Description |
| --- | --- |
| 1 | COM10 |
| 2 | OTO10 |
| 3 | COM11 |
| 4 | OTO11 |
| 5 | COM12 |
| 6 | OTO12 |
| 7 | COM13 |
| 8 | OTO13 |
| 9 | COM14 |
| 10 | OTO14 |
| 11 | COM15 |
| 12 | OTO15 |
| 13 | COM16 |
| 14 | OTO16 |
| 15 | COM17 |
| 16 | OTO17 |
| 17 | +5V |
| 18 | GND |

2.4 Pin Assignment of TB2

| Pin | Description |
| --- | --- |
| 1 | COM20 |
| 2 | OTO20 |
| 3 | COM21 |
| 4 | OTO21 |
| 5 | COM22 |
| 6 | OTO22 |
| 7 | COM23 |
| 8 | OTO23 |
| 9 | COM24 |
| 10 | OTO24 |
| 11 | COM25 |
| 12 | OTO25 |
| 13 | COM26 |
| 14 | OTO26 |
| 15 | COM27 |
| 16 | OTO27 |
| 17 | +5V |
|  | GND |

APPENDIX A

WARRANTY INFORMATION

1. Copyright

Copyright    1996   DECISION    COMPUTER INTERNATIONAL CO., LTD. all rights  reserved. No  part of POCKET ADDIO software and  manual may be reproduced, transmitted,  transcribed, or  translated into any language or  computer language,  in  any  form  or  by  any  means, electronic,  mechanical,  magnetic,  optical, chemical,  manual, or otherwise, without  the prior written permission of DECISION COMPUTER INTERNATIONAL CO.,  LTD.

Each  piece  of  POCKET  ADDIO   package permits  user to use POCKET ADDIO  only on  a single  computer, a registered user  may  use the program on a different computer, but  may not use the program on more than one computer at the same time.

Corporate  licensing  agreements   allow duplication  and distribution  of   specific number   of   copies  within   the   licensed institution.  Duplication of multiple  copies is not allowed except through execution of  a licensing   agreement.   Welcome   call   for

details.

2. Warranty Information

POCKET ADDIO warrants that for a  period of one year from the date of purchase (unless otherwise  specified  in the  warranty  card)

that   the   goods  supplied   will   perform according  to the specifications  defined  in the  user  manual.  Furthermore,  the  POCKET ADDIO   product  will be supplied  free  from defects  in materials and workmanship and  be fully functional under normal usage.

In the event of the failure of a  POCKET ADDIO  product within the specified  warranty period,  POCKET  ADDIO will, at  its option, replace  or repair the item at no  additional charge. This limited warranty does not  cover damage   resulting   from   incorrect    use, electrical    interference,   accident,    or modification of the product.

All  goods returned for warranty  repair must  have the serial number  intact.   Goods without  serial numbers attached will not  be

covered by the warranty.

Transportation costs for goods  returned must  be  paid by  the  purchaser.   Repaired goods  will be dispatched at the  expense  of

POCKET ADDIO.

To ensure that your POCKET ADDIO product is covered by the warranty provisions, it  is necessary that you return the Warranty card.

Under  this  Limited  Warranty,   POCKET ADDIO's obligations will be limited to repair or  replacement  only, of goods found  to  be defective  as  specified  above  during   the warranty period.  POCKET ADDIO is not  liable to the purchaser for any damages or losses of any kind, through the use of, or inability to use, the POCKET ADDIO product.

POCKET  ADDIO  reserves  the  right   to determine what constitutes warranty repair or replacement.

Return  Authorization: It is  necessary  that any returned goods are clearly marked with an RA  number  that has been  issued  by  POCKET ADDIO.     Goods   returned   without    this authorization will not be attended to.