CHAPTER 1

# INTRODUCTION

The 8255 I/O card is a programmable peripheral interface for PC/XT, PC/AT, 80386, 80486 or compatible. The interface card contains 8253 chips and 8255 chips, the 8253 chip provides programmable interval timer/counter functions and the 8255 chips provide programmable input/output functions.

- The features of the 8255 I/O card are:

*Programmable I/O control functions.

*Up to 48 I/O lines.

*Maximum of 2MHZ count rate.

*Three independent 16 bits counter.

*Support several operating modes, which are Programmable.

*Sixteen LED indicate when I/O is operating.

*Port address selectable.

- PACKAGE CONTENTS:

*8255/8253 I/O card.

*8255/8253 user's manual.

*8255/8253terminator board (option).

*Two expansion flat cables (option).

CHAPTER 2

# HARDWARE CONFIGURATION

## 2.1 Configuration for DIP Switch

Before you use the 8255 I/O card, you must ensure that the I/O address and the clock are set correctly. Observe the figure below the proper settings for the 8255 I/O card are described in the following:

1. I/O address

JP2: Short (default)

Select &H300 - &H30F as I/O port address. The I/O address correspond to three 8 bits ports and three counters are:

&H300: Port 1A input/output buffer.

&H301: Port 1B input/output buffer.

&H302: Port 1C input/output buffer.

&H303: Port 1 control register.

&H304: Port 2A input/output buffer.

&H305: Port 2B input/output buffer.

&H306: Port 2C input/output buffer.

&H307: Port 2 control register.

&H308: Counter 0 input/output buffer.

&H309: Counter 1 input/output buffer.

&H30A: Counter 2 input/output buffer.

&H30B: Counter control register

I/O address

JP2: Open

Select &H360 - &H36F as I/O port address. The I/O address correspond to three 8 bits ports and three counters are:

&H360: Port 1A input/output buffer.

&H361: Port 1B input/output buffer.

&H362: Port 1C input/output buffer.

&H363: Port 1 control register.

&H364: Port 2A input/output buffer.

&H365: Port 2B input/output buffer.

&H366: Port 2C input/output buffer.

&H367: Port 2 control register.

&H368: Counter 0 input/output buffer.

&H369: Counter 1 input/output buffer.

&H36A: Counter 2 input/output buffer.

&H36B: Counter control register.

2. Clock Selection

JP1-1 = Short (default) :

Select internal clock to counter 0.

JP1-2 = Short (default):

Select internal clock to counter 1.

JP1-3 = Short (default):

Select internal clock to counter 2.

JP1-1 = Open:

Select external clock to counter 0.

JP1-2 = Open:

Select external clock to counter 1.

JP1-3 = Open:

Select external clock to counter 2.

2.2 Hardware Installation

Your 8255 I/O card is design to be inserted in any available slot in your PC/XT or compatibles. In order to gain access to the expansion slots and the program switches on the main board, follow the steps listed below:

1.Set the 8255 I/O card switch.

2.Turn off all power of your computer and all peripheral devices before installing your 8255 I/O card.

3.Remove the cover of the computer.

4.Insert your pre-configured card into any available slot. Make sure your I/O card is firmly seated in the chosen slot.

5.Replace the cover of the computer.

6.You are now ready to use your 8255 I/O card for several Applications.

2.3 Pin Assignment

Port 1

| PIN | 11 GND 13 GND 15 PA1 17 CLKO 19 GATEO - - - - - - - - - - - - - 11 OUT2 13 CLK1 14 OUT1 17 PA5 19 PA7 - - - - - - - - - - - - - 21 PC6 23 PC4 25 PC1 27 PB7 29 PB6 - - - - - - - - - - - - - 31 PB5 33 PB4 35 PB3 37 +5V 39 +12v - - - - - - - - - - - - - | PIN | 12 GND 14 PA3 16 PA2 18 PA0 10 OUTO - - - - - - - - - - - - - 12 CLK2 14 GATE2 16 GATE1 18 PA4 20 PA6 - - - - - - - - - - - - - 22 PC7 24 PC5 26 PC0 28 PC2 30 PC3 - - - - - - - - - - - - - 32 PB0 34 PB1 36 PB2 38 -5V 40 -12V - - - - - - - - - - - - - |
| --- | --- | --- | --- |

Pin Assignment

Port 2

| PIN | 11 GND 13 GND 15 GND 17 GND 19 GND - - - - - - - - - - - - - 11 GND 13 PA0 15 PA2 17 PA4 19 PA6 - - - - - - - - - - - - - 21 PC7 23 PC5 25 PC0 27 PC2 29 PC3 - - - - - - - - - - - - - 31 PB0 33 PB1 35 PB2 37 +5V - +12V - - - - - - - - - - - - - | PIN | 12 GND 14 GND 16 GND 18 GND 10 GND - - - - - - - - - - - - - 12 GND 14 PA1 16 PA3 18 PA5 20 PA7 - - - - - - - - - - - - - 22 PC6 24 PC4 26 PC1 28 PB7 30 PB6 - - - - - - - - - - - - - 32 PB5 34 PB4 36 PB3 38 -5V 40 -12V - - - - - - - - - - - - - |
| --- | --- | --- | --- |

CHAPTER 3

# DIAGNOSTIC

1. BASIC version

100 REM 8255 I/O CARD TESTI

101 NG PROGRAM

110 SCREEN 0,0,0: WIDTH 80,25: KEY OFF: CLS

120 LOCATE 10,10: PRINT "8255 I/O CARD TESTING"

130 LOCATE 12,10: PRINT "TWO 8255 PORT A,B,C OUTPUT SQUARE WAVE"

140 LOCATE 14,10: PRINT "8253 COUNTER 0 DIVIDE BY 2"

150 LOCATE 16,10: PRINT " COUNTER 1 DIVIDE BY 50"

160 LOCATE 18,10: PRINT " COUNTER 2 DIVIDE BY 100"

250 REM 8253 TESTING

255 PORT = &H300

260 OUT PORT+11,&H36

270 OUT PORT+11,&H76

280 OUT PORT+11,&HB6

290 OUT PORT+8,&H2: OUT PORT+8,&H0

300 OUT PORT+9,&H32: OUT PORT+9,&H0

310 OUT PORT+10,&H64: OUT PORT+10,&H0

320 PORT=&H300

330 OUT PORT+3,&H80

340 A=0: GOSUB 500

345 FOR K= 0 TO 1000: NEXT K

350 A=&HFF: GOSUB 500

360 PORT=PORT+4

370 OUT PORT+3,&H80

380 A=0: GOSUB 500

385 FOR K=0 TO 1000: NEXT K

390 A=&HFF: GOSUB 500

400 GOTO 320

500 FOR I=0 TO 2

510 OUT PORT+I,A

520 NEXT I

530 RETURN

2. PASCAL version

Program diagnostic(input, output);

uses

Crt;

var

a, i, test : integer;

procedure subtest;

begin

for i := 0 to 2 do

port[test+i] := a;

end;

begin

{ 8255 I/O Card Testing Program }

clrscr;

gotoxy(10, 10);

writeln('8255 I/O CARD TESTING');

gotoxy(10, 12);

writeln('TWO 8255 PORT A,B,C OUTPUT SQUARE WAVE');

gotoxy(10, 14);

writeln('8253 COUNTER 0 DIVIDE BY 2');

gotoxy(10, 16);

writeln('8253 COUNTER 1 DIVIDE BY 50');

gotoxy(10, 18);

writeln('8253 COUNTER 2 DIVIDE BY 100');

{ 8253 Testing }

test := $300;

port[test+11] := $36;

port[test+11] := $76;

port[test+11] := $B6;

port[test+ 8] := $02; port[test+ 8] := $0;

port[test+ 9] := $32; port[test+ 9] := $0;

port[test+10] := $64; port[test+10] := $0;

repeat

test := $300;

port[test+3] := $80;

a := 0; subtest;

for i := 0 to 1000 do;

a := $ff; subtest;

test := test + 4;

port[test+3] := $80;

a := 0; subtest;

for i := 0 to 1000 do;

a := $ff; subtest

until keypressed;

end.

3. C version

#include <stdio.h>

#include <conio.h>

int test,i,a;

void subtest()

{

for(i=0;i<=2;i++) outportb(test+i,a);

}

main()

{

clrscr();

gotoxy(10,10);

puts("8255 I/O CARD TESTING");

gotoxy(10,12);

puts("TWO 8255 PORT A,B,C OUTPUT SQUARE WAVE");

gotoxy(10,14);

puts("8253 COUNTER 0 DIVIDE BY 2");

gotoxy(10,16);

puts("8253 COUNTER 1 DIVIDE BY 50");

gotoxy(10,18);

puts("8253 COUNTER 2 DIVIDE BY 100");

test = 0x300;

outportb(test+11,0x36);

outportb(test+11,0x76);

outportb(test+11,0xb6);

outportb(test+ 8,0x02);

outportb(test+ 8,0x00);

outportb(test+ 9,0x32);

outportb(test+ 9,0x00);

outportb(test+10,0x64);

outportb(test+10,0x00);

do {

test = 0x300;

outportb(test+3,0x80);

a = 0;

subtest();

for(i=0;i<=1000;i++);

a = 0xff;

subtest();

} while(!kbhit());

}

APPENDIX A

# TERMINATOR BOARD

The 8255terminator board provides expansion signal connection for convenience purpose. When power control on the normal condition, the LED will light. The layout of 8255terminator board is shown in the follows.

The signal assignment is shown in the follows.

1. TB1

| PORT 1 | PA0 |
| --- | --- |
| PORT 1 | PA1 |
| ###### PORT 1 | PA2 |
| PORT 1 | PA3 |
| PORT 1 | PA4 |
| PORT 1 | PA5 |
| PORT 1 | PA6 |
| PORT 1 | PA7 |
| PORT 1 | PB0 |
| PORT 1 | PB1 |
| PORT 1 | PB2 |
| PORT 1 | PB3 |
| PORT 1 | PB4 |
| PORT 1 | PB5 |
| PORT 1 | PB7 |

2. TB2

| PORT 1 | PC0 |
| --- | --- |
| PORT 1 | PC1 |
| PORT 1 | PC2 |
| PORT 1 | PC3 |
| PORT 1 | PC4 |
| PORT 1 | PC5 |
| PORT 1 | PC6 |
| PORT 1 | PC7 |
| PORT 2 | PA0 |
| PORT 2 | PA1 |
| PORT 2 | PA2 |
| PORT 2 | PA3 |
| PORT 2 | PA4 |
| PORT 2 | PA5 |
| PORT 2 | PA7 |

3. TB3

| PORT 2 | PB0 |
| --- | --- |
| PORT 2 | PB1 |
| PORT 2 | PB2 |
| PORT 2 | PB3 |
| PORT 2 | PB4 |
| PORT 2 | PB5 |
| PORT 2 | PB6 |
| PORT 2 | PB7 |
| PORT 2 | PC0 |
| PORT 2 | PC1 |
| PORT 2 | PC2 |
| PORT 2 | PC3 |
| PORT 2 | PC4 |
| PORT 2 | PC5 |
| PORT 2 | PC6 |
| PORT 2 | PC7 |

4. TB4

CLK0

GATE0

OUT0

CLK1

GATE1

OUT1

CLK2

GATE2

OUT2

GND1

GND2

+5V

-5V

GND1

GND1

GND2