CHAPTER 1

# INTRODUCTION

The 8255/8254 laboratory adapter is a programmable interface for PC/XT, PC/AT, PC/386, PC/486, Pentium or compatible computers. The interface adapter contains 8254 chip and 8255 chips, the 8254 chip provides programmable interval timer/counter functions and the 8255 chips provide programmable input/output functions. There are two type of cards, one contains many plated through-holes at the adapter for laboratory development and another is typical 8255/8254 adapter.

- The features of the 8255/8254 laboratory adapter are :

* Programmable I/O control functions.

* Up to 48 I/O lines.

* Maximum of 10MHZ count rate.

* Three independent 16 bits counter.

* Support several operating modes which are programmable.

* Port address selectable.

* Optional plated through-holes for laboratory development.

* Delay wait state selectable.

- Package contains:

* 8255/8254 laboratory adapter.

select one :

8255-LAB

plated through-holes are located

at the adapter.

8255-LED

LED at the adapter, and non-

plated through-holes.

* User's manual.

* 8255/8254 terminator board (option).

* Two expansion flat cables (option).

CHAPTER 2

# HARDWARE CONFIGURATION

## 2.1 Introduction

The three jumpers on the 8255/8254 laboratory adapter must be configured correctly in accordance with the system you using.

JP1 (Jumper 1)

Select internal or external clock for 8254.

JP2 (Jumper 2)

Determines the I/O address.

JP3 (Jumper 3)

Select wait state.

## 2.2 Jumper Settings

1. I/O address

The A9 to A4 jumper pins are used to set I/O port address, when the corresponding pin is short means 0, otherwise not short the pin means 1. The figure above set the I/O address to 2A0H.

When I/O address is selected, the three 8 bits ports and three counters are defined as follows.

Base + 0 : Port 1A input/output buffer.

Base + 1 : Port 1B input/output buffer.

Base + 2 : Port 1C input/output buffer.

Base + 3 : Port 1 control register.

Base + 4 : Port 2A input/output buffer.

Base + 5 : Port 2B input/output buffer.

Base + 6 : Port 2C input/output buffer.

Base + 7 : Port 2 control register.

Base + 8 : Counter 0 input/output buffer.

Base + 9 : Counter 1 input/output buffer.

Base + A : Counter 2 input/output buffer.

Base + B : Counter control register.

2. Clock Selection

1. JP1-1 Short (default)

Select 4.0 MHZ internal clock to counter 0.

2. JP1-2 Short (default)

Select 4.0 MHZ internal clock to counter 1.

3. JP1-3 Short (default)

Select 4.0 MHZ internal clock to counter 2.

4. JP1-1 Open

Select external clock to counter 0.

Maximum 10 MHZ.

5. JP1-2 Open

Select external clock to counter 1.

Maximum 10 MHZ.

6. JP1-3 Open

Select external clock to counter 2.

Maximum 10 MHZ

3. Delay Wait State

pin no      description

1        no wait

2        delay 1 CPU clock

3        delay 2 CPU clock

## 2.3 Hardware Installation

Your 8255/8254 laboratory adapter is designed to be inserted in any available slot in your PC/XT, PC/286, PC/386, PC/486, Pentium or compatibles. In order to gain access to the expansion slots, follow the steps listed below:

1. Set the jumpers of 8255/8254 laboratory adapter.

2. Turn off all power of your computer and all peripheral devices before installing your 8255/8254 laboratory adapter.

3. Remove the cover of the computer.

4. Insert your preconfigured card into any available slot. Make sure your I/O adapter is firmly seated in the chosen slot.

5. Replace the cover of the computer.

## 2.4 Pin Assignment

Port 1

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| 1 3 5 7 9 | GND GND PA1 CLK0 GATE0 | 2 4 6 8 10 | GND PA3 PA2 PA0 OUT0 |
| 11 13. 15 17 19 | OUT2 CLK1 OUT1 PA5 PA7 | 12 14 16 18 20 | CLK2 GATE2 GATE1 PA4 PA6 |
| 21 23 25 27 29 | PC6 PC4 PC1 PB7 PB6 | 22 24 26 28 30 | PC7 PC5 PC0 PC2 PC3 |
| 31 33 35 37 39 | PB5 PB4 PB3 +5V +12V | 32 34 36 38 40 | PB0 PB1 PB2 -5V -12V |

Port 2

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| 1 3 5 7 9 | GND GND GND GND GND | 2 4 6 8 10 | GND GND GND GND GND |
| 11 13. 15 17 19 | GND PA0 PA2 PA4 PA6 | 12 14 16 18 20 | GND PA1 PA3 PA5 PA7 |
| 21 23 25 27 29 | PC7 PC5 PC0 PC2 PC3 | 22 24 26 28 30 | PC6 PC4 PC1 PB7 PB6 |
| 31 33 35 37 39 | PB0 PB1 PB2 +5V +12V | 32 34 36 38 40 | PB5 PB4 PB3 -5V -12V |

CHAPTER 3

# DIAGNOSTIC

1. BASIC version

100 REM 8255 I/O CARD TESTI

101 NG PROGRAM

110 SCREEN 0,0,0: WIDTH 80,25: KEY OFF: CLS

120 LOCATE 10,10: PRINT "8255 I/O CARD TESTING"

130 LOCATE 12,10: PRINT "TWO 8255 PORT A,B,C OUTPUT SQUARE WAVE"

140 LOCATE 14,10: PRINT "8253 COUNTER 0 DIVIDE BY 2"

150 LOCATE 16,10: PRINT " COUNTER 1 DIVIDE BY 50"

160 LOCATE 18,10: PRINT " COUNTER 2 DIVIDE BY 100"

250 REM 8253 TESTING

255 PORT = &H1B0

260 OUT PORT+11,&H36

270 OUT PORT+11,&H76

280 OUT PORT+11,&HB6

290 OUT PORT+8,&H2: OUT PORT+8,&H0

300 OUT PORT+9,&H32: OUT PORT+9,&H0

310 OUT PORT+10,&H64: OUT PORT+10,&H0

320 PORT=&H1B0

330 OUT PORT+3,&H80

340 A=0: GOSUB 500

345 FOR K= 0 TO 1000: NEXT K

350 A=&HFF: GOSUB 500

360 PORT=PORT+4

370 OUT PORT+3,&H80

380 A=0: GOSUB 500

385 FOR K=0 TO 1000: NEXT K

390 A=&HFF: GOSUB 500

400 GOTO 320

500 FOR I=0 TO 2

510 OUT PORT+I,A

520 NEXT I

530 RETURN

2. PASCAL version

Program diagnostic(input, output);

uses

Crt;

var

a, i, test : integer;

procedure subtest;

begin

for i := 0 to 2 do

port[test+i] := a;

end;

begin

{ 8255 I/O Card Testing Program }

clrscr;

gotoxy(10, 10);

writeln('8255 I/O CARD TESTING');

gotoxy(10, 12);

writeln('TWO 8255 PORT A,B,C OUTPUT SQUARE WAVE');

gotoxy(10, 14);

writeln('8253 COUNTER 0 DIVIDE BY 2');

gotoxy(10, 16);

writeln('     COUNTER 1 DIVIDE BY 50');

gotoxy(10, 18);

writeln('     COUNTER 2 DIVIDE BY 100');

{ 8253 Testing }

test := $1B0;

port[test+11] := $36;

port[test+11] := $76;

port[test+11] := $B6;

port[test+ 8] := $02; port[test+ 8] := $0;

port[test+ 9] := $32; port[test+ 9] := $0;

port[test+10] := $64; port[test+10] := $0;

repeat

test := $1B0;

port[test+3] := $80;

a := 0; subtest;

for i := 0 to 1000 do;

a := $ff; subtest;

test := test + 4;

port[test+3] := $80;

a := 0; subtest;

for i := 0 to 1000 do;

a := $ff; subtest

until keypressed;

end.

3. C version

#include <stdio.h>

#include <conio.h>

int test,i,a;

void subtest()

{

for(i=0;i<=2;i++) outportb(test+i,a);

}

main()

{

clrscr();

gotoxy(10,10);

puts("8255 I/O CARD TESTING");

gotoxy(10,12);

puts("TWO 8255 PORT A,B,C OUTPUT SQUARE WAVE");

gotoxy(10,14);

puts("8253 COUNTER 0 DIVIDE BY 2");

gotoxy(10,16);

puts("8253 COUNTER 1 DIVIDE BY 50");

gotoxy(10,18);

puts("8253 COUNTER 2 DIVIDE BY 100");

test = 0x1b0;

outportb(test+11,0x36);

outportb(test+11,0x76);

outportb(test+11,0xb6);

outportb(test+ 8,0x02);

outportb(test+ 8,0x00);

outportb(test+ 9,0x32);

outportb(test+ 9,0x00);

outportb(test+10,0x64);

outportb(test+10,0x00);

do {

test = 0x1b0;

outportb(test+3,0x80);

a = 0;

subtest();

for(i=0;i<=1000;i++);

a = 0xff;

subtest();

} while(!kbhit());

}

APPENDIX A

# WINDOWS LIBRARY

The port.dll file provides I/O functions for 8255/8253 I/O cards, each function is specified as follows.  Where port and data are defined as integer.

int DCI_inb (port)

Read one byte from port address.

int DCI_inw (port)

Read one word (16 bits) from port address.

DCI_outb (port, data)

Write data (byte) to port.

DCI_outw (port,data)

Write data (word) to port.

Under visual BASIC, to declare the function and call the function is specified as follows.

General declaration

Declare sub DCI_outb Lib “a:\port.dll”

(ByVal port As Integer, ByVal data As Integer)

Declare Function DCI_inb Lib “a:\port.dll”

(ByVal port As Integer) As Integer.

Library call

DCI_outb port, data		/*for output*/

Res = DCI_inb (port+4)	/*for input*/

Under Borland C :

General declaration

HModule = LoadLibrary (“a:\port.dll”)

myin = GetProcAddress (hModule, “DCI_inb”);

myout = GetProcAddress (hModule,”DCI_outb”)

Library call

myout (port, odata);

idata = myin (port+4);

We provide sample program in the distrigution diskette. For Visual BASIC, please refer “8255_2.mak” and “8255_2.frm” files. For Borland C, please refer “8255_2.c” and “8255_2.def” files.

APPENDIX B

# TERMINATOR BOARD

The 8255 terminator board provides expansion signal connection for convenience purpose. When power control on the normal condition, the LED will light. The layout of 8255 terminator board is shown in the follows.

The signal assignment is shown in the follows.

1. TB1

| PORT 1 | PA0 |
| --- | --- |
| PORT 1 | PA1 |
| ###### PORT 1 | PA2 |
| PORT 1 | PA3 |
| PORT 1 | PA4 |
| PORT 1 | PA5 |
| PORT 1 | PA6 |
| PORT 1 | PA7 |
| PORT 1 | PB0 |
| PORT 1 | PB1 |
| PORT 1 | PB2 |
| PORT 1 | PB3 |
| PORT 1 | PB4 |
| PORT 1 | PB5 |
| PORT 1 | PB7 |

2. TB2

| PORT 1 | PC0 |
| --- | --- |
| PORT 1 | PC1 |
| PORT 1 | PC2 |
| PORT 1 | PC3 |
| PORT 1 | PC4 |
| PORT 1 | PC5 |
| PORT 1 | PC6 |
| PORT 1 | PC7 |
| PORT 2 | PA0 |
| PORT 2 | PA1 |
| PORT 2 | PA2 |
| PORT 2 | PA3 |
| PORT 2 | PA4 |
| PORT 2 | PA5 |
| PORT 2 | PA7 |

3. TB3

| PORT 2 | PB0 |
| --- | --- |
| PORT 2 | PB1 |
| PORT 2 | PB2 |
| PORT 2 | PB3 |
| PORT 2 | PB4 |
| PORT 2 | PB5 |
| PORT 2 | PB6 |
| PORT 2 | PB7 |
| PORT 2 | PC0 |
| PORT 2 | PC1 |
| PORT 2 | PC2 |
| PORT 2 | PC3 |
| PORT 2 | PC4 |
| PORT 2 | PC5 |
| PORT 2 | PC6 |
| PORT 2 | PC7 |

4. TB4

CLK0

GATE0

OUT0

CLK1

GATE1

OUT1

CLK2

GATE2

OUT2

GND1

GND2

+5V

-5V

GND1

GND1

GND2

APPENDIX C

# WARRANTY INFORMATION

## C.1 Copyright

Copyright 1994,1995 DECISION COMPUTER INTERNATIONAL CO., LTD. all rights reserved. No part of SmartLab software and manual may be reproduced, transmitted, transcribed, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical, manual, or otherwise, without the prior written permission of DECISION COMPUTER INTERNATIONAL CO., LTD.

Each piece of SmartLab package permits user to use SmartLab only on a single computer, a registered user may use the program on a different computer, but may not use the program on more than one computer at the same time.

Corporate licensing agreements allow duplication and distribution of specific number of copies within the licensed institution. Duplication of multiple copies is not allowed except through execution of a licensing agreement. Welcome call for details.

## C.2 Warranty Information

SmartLab warrants that for a period of one year from the date of purchase (unless otherwise specified in the warranty card) that the goods supplied will perform according to the specifications defined in the user manual. Furthermore that the SmartLab product will be supplied free from defects in materials and workmanship and be fully functional under normal usage.

In the event of the failure of a SmartLab product within the specified warranty period, SmartLab will, at its option, replace or repair the item at no additional charge. This limited warranty does not cover damage resulting from incorrect use, electrical interference, accident, or modification of the product.

All goods returned for warranty repair must have the serial number intact. Goods without serial numbers attached will not be covered by the warranty.

Transportation costs for goods returned must be paid by the purchaser. Repaired goods will be dispatched at the expense of

SmartLab.

To ensure that your SmartLab product is covered by the warranty provisions, it is necessary that you return the Warranty card.

Under this Limited Warranty, SmartLab's obligations will be limited to repair or replacement only, of goods found to be defective as specified above during the warranty period. SmartLab is not liable to the purchaser for any damages or losses of any kind, through the use of, or inability to use, the SmartLab product.

SmartLab reserves the right to determine what constitutes warranty repair or replacement.

Return Authorization: It is necessary that any returned goods are clearly marked with an RA number that has been issued by SmartLab. Goods returned without this authorization will not be attended to.