CHAPTER 1

# INTRODUCTION

The multiple 8255 I/O card is programmable peripheral interface for PC/XT, PC/AT or compatibles. The interface card provides 192 parallel input output interface for PC/XT, PC/AT or compatibles. The interface card provides 192 parallel input output (PIO) programmable line, and six independent 16 bits counter/clock.

- The features of the Multi-8255 adapter are:

- Programmable I/O control functions.

- Up to 192 parallel I/O lines.

- Maximum of 2.35MHZ count rate.

- Six independent 16 bits counters.

- Support several operating modes, which are programmable.

- Port address selectable.

CHAPTER 2

# HARDWARE CONFIGURATION

## 2.1 Configuration for jumpers setting

Before you use the multi-8255 I/O card, you must ensure that the I/O address and the clock are set correctly. Observe the figure bellows the proper settings for the multi-8255 I/O card are described in the following:

1. I/O Address

The JP1 is used to select I/O port address, the address mapping of 8255 chips and 8253 chips are shown in the followings:

Address Mapping

| Chip No. | JP1 short | JP1 open |
| --- | --- | --- |
| 8255 (1) | 280-283 | 300-303 |
| 8255 (2) | 284-287 | 304-307 |
| 8255 (3) | 288-28B | 308-30B |
| 8255 (4) | 28C-28F | 30C-30F |
| 8255 (5) | 290-293 | 310-313 |
| 8255 (6) | 294-297 | 314-317 |
| 8255 (7) | 298-29B | 318-31B |
| 8255 (8) | 29C-29F | 31C-31F |
| 8255 (1) | 2A0-2A3 | 320-323 |
| 8255 (2) | 2A8-2AB | 328-32B |

JP1: Open

Select &H300 - &H32B as I/O port address. The I/O functions are shown in the following:

&H300: Port 1A input/output buffer.

&H301: Port 1B input/output buffer.

&H302: Port 1C input/output buffer.

&H303: Port 1 control register.

&H304: Port 2A input/output buffer.

&H305: Port 2B input/output buffer.

&H306: Port 2C input/output buffer.

&H307: Port 2 input/output buffer.

&H308: Port 3A input/output buffer.

&H309: Port 3B input/output buffer.

&H30A: Port 3C input/output buffer.

&H30B: Port 3 control register.

&H30C: Port 4A input/output buffer.

&H30D: Port 4B input/output buffer.

&H30E: Port 4C input/output buffer.

&H30F: Port 4 control register.

&H310: Port 5A input/output buffer.

&H311: Port 5B input/output buffer.

&H312: Port 5C input/output buffer.

&H313: Port 5 control register.

&H314: Port 6A input/output buffer.

&H315: Port 6B input/output buffer.

&H316: Port 6C input/output buffer.

&H317: Port 6 control register.

&H318: Port 7A input/output buffer.

&H319: Port 7B input/output buffer.

&H31A: Port 7C input/ output buffer.

&H31B: Port 7 control register.

&H31C: Port 8A input/output buffer.

&H31D: Port 8B input/output buffer.

&H31E: Port 8C input/output buffer.

&H31F: Port 8 control register.

&H320: Counter 1A input/output buffer.

&H321: Counter 1B input/output buffer.

&H322: Counter 1C input/output buffer.

&H323: Counter 1 control register.

&H328: Counter 2A input/output buffer.

&H329: Counter 2B input/output buffer.

&H32A: Counter 2C input/output buffer.

&H32B: Counter 2 control register.

&H308: Port 3A input/output buffer.

&H309: Port 3B input/output buffer.

&H30A: Port 3C input/output buffer.

JP1: Close (Default)

Select &H280 - &H2AB as I/O port address. The I/O functions are shown in the followings:

&H280: Port 1A input/output buffer.

&H281: Port 1B input/output buffer.

&H282: Port 1C input/output buffer.

&H283: Port 1 control register.

&H284: Port 2A input/output buffer.

&H285: Port 2B input/output buffer.

&H286: Port 2C input/output buffer.

&H287: Port 2 control register.

&H288: Port 3A input/output buffer.

&H289: Port 3B input/output buffer.

&H28B: Port 3 control register.

&H28C: Port 4A input/output buffer.

&H28D: Port 4B input/output buffer.

&H28E: Port 4C input/output buffer.

&H28F: Port 4 control register.

&H290: Port 5A input/output buffer.

&H291: Port 5B input/output buffer.

&H292: Port 5C input/output buffer.

&H293: Port 5 control register.

&H294: Port 6A input/output buffer.

&H295: Port 6B input/output buffer.

&H296: Port 6C input/output buffer.

&H297: Port 6 control register.

&H298: Port 7A input/output buffer.

&H299: Port 7B input/output buffer.

&H29A: Port 7C input/output buffer.

&H29B: Port 7 control register.

&H29C: Port 8A input/output buffer.

&H29D: Port 8B input/output buffer.

&H29E: Port 8C input/output buffer.

&H29F: Port 8 control register.

&H2A0: Counter 1A input/output buffer.

&H2A1: Counter 1B input/output buffer.

&H2A2: Counter 1C input/output buffer.

&H2A3: Counter 1 control register.

&H2A8: Counter 2A input/output buffer.

&H2A9: Counter 2B input/output buffer.

&H2AA: Counter 2C input/output buffer.

&H2AB: Counter 2 control register.

2. Clock Selection

JP2: JP2 is used to select clock of 8253 (1).

JP2-1 = Short (default):

Select internal clock to counter 1A

JP2-2 = Short (default):

Select internal clock to counter 1B.

JP2-3 = Short (default):

Select internal clock to counter 1C.

JP2-1 = Open:

Select external clock to counter 1A.

JP2-2 = Open:

Select external clock to counter 1B.

JP2-3 = Open:

Select external clock to counter 1C.

JP3: JP3 is used to select clock of 8253 (2).

JP2-1 = Short (default):

Select internal clock to counter 2A.

JP2-2 = Short (default):

Select internal clock to counter 2B.

JP2-3 = Short (default):

Select internal clock to counter 2C.

JP2-1 = Open:

Select external clock to counter 2A.

JP2-2 = Open:

Select external clock to counter 2B.

JP2-3 = Open:

Select external clock to counter 2C.

2.2 Hardware Installation

Your multi-8255 I/O card is designed to be inserted in any available slot in your PC/XT, PC/AT, or compatibles. In order to gain access to the expansion slots and the programs switches on the main board, follow the steps listed below:

1. Set the multi-8255 I/O card switch.

2. Turn off all power of your computer and all peripheral devices before installing your multi-8255 I/O card.

3. Remove the cover of the computer.

4. Insert your reconfigured card into any available slot. Make sure your I/O card is firmly seated in the chosen slot.

5. Replace the cover of the computer.

6. You are now ready to use your multi-8255 I/O card for several applications.

2.3 Pin Assignment

There are four 50 pins I/O connectors, where CN1, CN2, CN3, CN4 corresponds to 8255 I/O ports, and CN5 corresponds to 8253 counter.

CONNECTOR CN1: 8255 (U6) and 8255 (U7)

| PIN; 11 1PA0 12 1PA1 13 1PA2 14 1PA3 15 1PA4 - - - - - - - - - - - - - 16 1PA5 17 1PA6 18 1PA7 19 1PB0 10 1PB1 - - - - - - - - - - - - - 11 1PB2 12 1PB3 13 1PB4 14 1PB5 15 1PB6 - - - - - - - - - - - - - 16 1PB7 17 1PC0 18 1PC1 19 1PC2 20 1PC3 - - - - - - - - - - - - - 21 1PC4 22 1PC5 23 1PC6 24 1PC7 25 GND. | PIN; 26 GND 27 2PA0 28 2PA1 29 2PA2 30 2PA3 - - - - - - - - - - - - - 31 2PA4 32 2PA5 33 2PA6 34 2PA7 35 2PB0 - - - - - - - - - - - - - 36 2PB1 37 2PB2 38 2PB3 39 2PB4 40 2PB5 - - - - - - - - - - - - - 41 2PB6 42 2PB7 43 2PC0 44 2PC1 45 2PC2 - - - - - - - - - - - - - 46 2PB3 47 2PB4 48 2PC5 49 2PC6 50 2PC7. |
| --- | --- |

CONNECTOR CN2: 8255 (U8) and 8255 (U9)

| PIN; 11 3PA0 12 3PA1 13 3PA2 14 3PA3 15 3PA4 - - - - - - - - - - - - - 16 3PA5 17 3PA6 18 3PA7 19 3PB0 10 3PB1 - - - - - - - - - - - - - 11 3PB2 12 3PB3 13 3PB4 14 3PB5 15 3PB6 - - - - - - - - - - - - - 16 3PB7 17 3PC0 18 3PC1 19 3PC2 20 3PC3 - - - - - - - - - - - - - 21 3PC4 22 3PC5 23 3PC6 24 3PC7 25 GND. | PIN; 26 GND 27 4PA0 28 4PA1 29 4PA2 30 4PA3 - - - - - - - - - - - - - 31 4PA4 32 4PA5 33 4PA6 34 4PA7 35 4PB0 - - - - - - - - - - - - - 36 4PB1 37 4PB2 38 4PB3 39 4PB4 40 4PB5 - - - - - - - - - - - - - 41 4PB6 42 4PB7 43 4PC0 44 4PC1 45 4PC2 - - - - - - - - - - - - - 46 4PB3 47 4PB4 48 4PC5 49 4PC6 50 4PC7. |
| --- | --- |

CONNECTOR CN3: 8255 (U1) and 8255 (U2)

| PIN; 11 5PA0 12 5PA1 13 5PA2 14 5PA3 15 5PA4 - - - - - - - - - - - - - 16 5PA5 17 5PA6 18 5PA7 19 5PB0 10 5PB1 - - - - - - - - - - - - - 11 5PB2 12 5PB3 13 5PB4 14 5PB5 15 5PB6 - - - - - - - - - - - - - 16 5PB7 17 5PC0 18 5PC1 19 5PC2 20 5PC3 - - - - - - - - - - - - - 21 5PC4 22 5PC5 23 5PC6 24 5PC7 25 GND. | PIN; 26 GND 27 6PA0 28 6PA1 29 6PA2 30 6PA3 - - - - - - - - - - - - - 31 6PA4 32 6PA5 33 6PA6 34 6PA7 35 6PB0 - - - - - - - - - - - - - 36 6PB1 37 6PB2 38 6PB3 39 6PB4 40 6PB5 - - - - - - - - - - - - - 41 6PB6 42 6PB7 43 6PC0 44 6PC1 45 6PC2 - - - - - - - - - - - - - 46 6PB3 47 6PB4 48 6PC5 49 6PC6 50 6PC7. |
| --- | --- |

CONNECTOR CN4: 8255 (U3) and 8255 (U4)

| PIN; 11 7PA0 12 7PA1 13 7PA2 14 7PA3 15 7PA4 - - - - - - - - - - - - - 16 7PA5 17 7PA6 18 7PA7 19 7PB0 10 7PB1 - - - - - - - - - - - - - 11 7PB2 12 7PB3 13 7PB4 14 7PB5 15 7PB6 - - - - - - - - - - - - - 16 7PB7 17 7PC0 18 7PC1 19 7PC2 20 7PC3 - - - - - - - - - - - - - 21 7PC4 22 7PC5 23 7PC6 24 7PC7 25 GND. | PIN; 26 GND 27 8PA0 28 8PA1 29 8PA2 30 8PA3 - - - - - - - - - - - - - 31 8PA4 32 8PA5 33 8PA6 34 8PA7 35 8PB0 - - - - - - - - - - - - - 36 8PB1 37 8PB2 38 8PB3 39 8PB4 40 8PB5 - - - - - - - - - - - - - 41 8PB6 42 8PB7 43 8PC0 44 8PC1 45 8PC2 - - - - - - - - - - - - - 46 8PB3 47 8PB4 48 8PC5 49 8PC6 50 8PC7. |
| --- | --- |

CONNECTOR CN5: 8255 (U5) and 8255 (U6)

| PIN; 11 1PA0 12 1ACK 13 1AGAT 14 1AOUT 15 GND - - - - - - - - - - - - - 16 1BCK 17 1BGAT 18 1BOUT 19 GND 10 1CCK - - - - - - - - - - - - - 11 1CGAT 12 1COUT 13 GND 14 2ACK 15 2AGAT - - - - - - - - - - - - - 16 2AOUT 17 GND 18 2BCK 19 1BGAT 20 2BOUT - - - - - - - - - - - - - 21 GND 22 2CCK 23 2CGAT 24 2COUT 25 GND. | PIN; 26 5V 27 GND 28 -5V 29 GND 30 +12V - - - - - - - - - - - - - 31 GND 32 -12V 33 GND 34 GND. |
| --- | --- |