CHAPTER 1

INTRODUCTION

The dual stepping motor board is used to drive stepping motor, and provides two stepping motors control circuit.  To drive the stepping motor, user can  connect Pocket 16 DO box to dual stepping motor board, then connect stepping motor to it.  The pocket 16 DO box is no need to occupy any slot of your PC system, it is suitable for any PC that contains a parallel printer port.  Each stepping motor controller provides four output signals and receives six bits control string to drive stepping motor.

The  features of the dual stepping motor board are:

- Support two stepping motor control circuit.

- Voltage range: 14 to 28V.

- Maximum current: 1 A.

- The stepping motor is driven by pocket 16 DO box which is connected to parallel print port.

- Support half step and full step revolution.

- Operating temperature 0 to 60 ºC.

- Storage temperature –20 to 70 degree ºC.

- Humidity 5% to 95% in non-condensing.

The package contains:

- Dual stepping motor board.

- User’s manual.

- Pocket 16 DO box and manual (optional).

CHAPTER 2

HARDWARE INSTALLATION

Your dual stepping motor board is designed to be inserted to the pocket 16 DO box, please follow the steps listed below:

- Turn off  all power to your computer and all peripheral devices before installing your dual stepping motor board.

- Connect pocket 16 DO box to printer port, then plug in power adapter to pocket 16 DO box.

- Connect dual stepping motor board to pocket 16 DO box, then plug in power to stepping motor board.

- Connect stepping motor to dual stepping motor board.

- Turn on the computer and power adapter.

CHAPTER 3

HARDWARE CONFIGURATIONS

Before you use the dual stepping motor board, you must ensure that the power and signal are set correctly.  The proper settings for the dual stepping motor board are described in the following:

  - TB1

The TB1 is used to connect the control signal of stepping motor, it will rotate clockwise if connect is correct, otherwise it will rotate counterclockwise or shaking.  Please connect it correctly.

  - TB2

The TB1 is used to connect the control signal of stepping motor, it will rotate clockwise if connect is correct, otherwise it will rotate counterclockwise or shaking.  Please connect it correctly.

  - TB3 (Power)

The TB3 is used to supple 14V to 28V power.

  - Pin Assignment of Pocket 16 DO

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| 1 | GND | 14 | +5V |
| 2 | DO10 | 15 | GND |
| 3 | DO11 | 16 | GND |
| 4 | DO12 | 17 | GND |
| 5 | DO13 | 18 | GND |
| 6 | DO14 | 19 | GND |
| 7 | DO15 | 20 | GND |
| 8 | DO16 | 21 | DO23 |
| 9 | DO17 | 22 | DO24 |
| 10 | DO20 | 23 | DO25 |
| 11 | DO21 | 24 | DO27 |
| 12 | DO22 | 25 | +5V |
| 13 | GND |  |  |

CHAPTER 4

SOFTWARE  PROGRAMMING

In the distribution diskette, we provide C, PASCAL, ASSEMBLY, and BASIC examples to show how to write software to control stepping motor.  It is easy to run under MS/DOS or Windows environment.

  - Control Signal

The DO10 to DO17 are used to control stepping motor that was connected to TB1.  The DO20 to DO27 are used to control stepping motor that was connected to TB2.

  - Software Programming

There are half step and full step control string to control stepping motor.  To control stepping motor, please follow the control sequence that specifies in the below, and repeat output these control sequences from pocket 16DO box.

The  control sequence of half method are:

DO17 DO16 DO15 DO14 DO13 DO12 DO11 DO10

DO27 DO26 DO25 DO24 DO23 DO22 DO21 DO20

1. 0 0 1 1 0 1 1 1

2. 0 0 1 1 0 1 1 0

3. 0 0 1 0 1 1 1 0

4. 0 0 1 0 0 1 1 0

5. 0 0 1 0 0 1 0 1

6. 0 0 1 0 0 1 0 0

7. 0 0 1 1 1 1 0 0

8. 0 0 1 1 0 1 0 0

The control sequence of full step method are:

DO17 DO16 DO15 DO14 DO13 DO12 DO11 DO10

DO27 DO26 DO25 DO24 DO23 DO22 DO21 DO20

1. 0 0 0 0 0 0 0 0

2. 0 0 0 0 0 0 1 0

3. 0 0 0 1 0 0 1 0

4. 0 0 0 1 0 0 0 0

APPENDIX A

WARRANTY INFORMATION

1. Copyright

Copyright    1996   DECISION    COMPUTER INTERNATIONAL CO., LTD. all rights  reserved. No  part of POCKET ADDIO software and  manual may be reproduced, transmitted,  transcribed, or  translated into any language or  computer language,  in  any  form  or  by  any  means, electronic,  mechanical,  magnetic,  optical, chemical,  manual, or otherwise, without  the prior written permission of DECISION COMPUTER INTERNATIONAL CO.,  LTD.

Each  piece  of  POCKET  ADDIO   package permits  user to use POCKET ADDIO  only on  a single  computer, a registered user  may  use the program on a different computer, but  may not use the program on more than one computer at the same time.

Corporate  licensing  agreements   allow duplication  and distribution  of   specific number   of   copies  within   the   licensed institution.  Duplication of multiple  copies is not allowed except through execution of  a licensing   agreement.   Welcome   call   for

details.

2. Warranty Information

POCKET ADDIO warrants that for a  period of one year from the date of purchase (unless otherwise  specified  in the  warranty  card)

that   the   goods  supplied   will   perform according  to the specifications  defined  in the  user  manual.  Furthermore,  the  POCKET ADDIO   product  will be supplied  free  from defects  in materials and workmanship and  be fully functional under normal usage.

In the event of the failure of a  POCKET ADDIO  product within the specified  warranty period,  POCKET  ADDIO will, at  its option, replace  or repair the item at no  additional charge. This limited warranty does not  cover damage   resulting   from   incorrect    use, electrical    interference,   accident,    or modification of the product.

All  goods returned for warranty  repair must  have the serial number  intact.   Goods without  serial numbers attached will not  be

covered by the warranty.

Transportation costs for goods  returned must  be  paid by  the  purchaser.   Repaired goods  will be dispatched at the  expense  of

POCKET ADDIO.

To ensure that your POCKET ADDIO product is covered by the warranty provisions, it  is necessary that you return the Warranty card.

Under  this  Limited  Warranty,   POCKET ADDIO's obligations will be limited to repair or  replacement  only, of goods found  to  be defective  as  specified  above  during   the warranty period.  POCKET ADDIO is not  liable to the purchaser for any damages or losses of any kind, through the use of, or inability to use, the POCKET ADDIO product.

POCKET  ADDIO  reserves  the  right   to determine what constitutes warranty repair or replacement.

Return  Authorization: It is  necessary  that any returned goods are clearly marked with an RA  number  that has been  issued  by  POCKET ADDIO.     Goods   returned   without    this authorization will not be attended to.