8255 I/O CARD DOS TEST SOFTWARE SOURCE CODE

/*  This is the QC program for 8255/8253 I/O CARD

Author : Johnny Tseng

Date : 1999/10/02   */

#include <dos.h>

#include <stdio.h>

#include <conio.h>

/* set default value for ioport, CNTR */

unsigned int ioport = 0;

void test8255loopback (void)

{

int loop, i, c, step = 0;

unsigned char x1, x2, x3, y1, y2, y3, z1, z2, z3;

unsigned long x;

unsigned long y;

unsigned duration = 64;

if (!ioport)

return;

gotoxy (10, 16);

outportb (ioport + 3, 0x80);

outportb (ioport + 7, 0x9b);

for (loop = 0; loop < 100; loop ++) {

for (i = 0; i < 24; i++) {

x = (unsigned long)1 << i;

x1 = x & 0x00ff;

x2 = (x >> 8) & 0x00ff;

x3 = (x >> 16) & 0x00ff;

y = 0x00800000 >> i;

y1 = y & 0x00ff;

y2 = y >> 8;

y3 = y >> 16;

outportb (ioport, x1);

outportb (ioport + 1, x2);

outportb (ioport + 2, x3);

delay (duration);

z1 = inportb (ioport + 4);

z2 = inportb (ioport + 5);

z3 = inportb (ioport + 6);

gotoxy (3, 16);

cprintf("Loop: %3d  Testing Pin:%2d  Output: %02X%02X%02X  Input:%02X%02X%02X  Correct:%02X%02X%02X",

loop + 1, i, x3, x2, x1, z3, z2, z1, y3, y2, y1);

if (z1 != y1 || z2 != y2 || z3 != y3) {

cputs (" Error ");

c = getch ();

if (c == 27)

break;

}

else {

cputs ("  OK   ");

//c = getch ();

}

if (kbhit () || step) {

c = getch ();

if (c == 27)

break;

else if (c == ' ')

step = !step;

else if (c == '+') {

if (duration > 1)

duration >>= 1;

}

else if (c == '-') {

if (duration < 512)

duration <<= 1;

}

else if (!step)

getch ();

}

}

if (c == 27)

break;

}

gotoxy (3, 17);

cputs("Press any key when ready");

if (kbhit ())

getch();

}

void dumpbits (unsigned long x)

{

int i;

for (i = 0; i < 24; i++) {

putch (((0x00800000 >> i) & x)? '1' : '0');

}

}

void test8255output (void)

{

unsigned long x, loop = 0L;

int i, flag, iop, step = 0;

unsigned char x1, x2, x3, c = 0;

unsigned duration = 64;

outportb (ioport + 3, 0x80);

outportb (ioport + 7, 0x80);

outportb (ioport, 0);

outportb (ioport + 1, 0);

outportb (ioport + 2, 0);

outportb (ioport + 4, 0);

outportb (ioport + 5, 0);

outportb (ioport + 6, 0);

for (flag = 0;;flag = !flag) {

outportb (iop, 0);

outportb (iop + 1, 0);

outportb (iop + 2, 0);

if (flag) {

iop = ioport + 4;

} else {

iop = ioport;

}

for (i = 0, x = (unsigned long)1; i < 24; i++, x <<= 1, loop++) {

x1 = x & 0x00ff;

x2 = (x >> 8) & 0x00ff;

x3 = (x >> 16) & 0x00ff;

outportb (iop, x1);

outportb (iop + 1, x2);

outportb (iop + 2, x3);

gotoxy (3, 16);

cprintf("Loop: %8ld : ", loop);

if (flag) {

dumpbits (x);

dumpbits (0);

} else {

dumpbits (0);

dumpbits (x);

}

delay (duration);

if (kbhit () || step) {

c = getch ();

if (c == 27)

break;

else if (c == ' ')

step = !step;

else if (c == '+') {

if (duration > 1)

duration >>= 1;

}

else if (c == '-') {

if (duration < 512)

duration <<= 1;

}

else if (!step)

getch ();

}

} // for (i)

if (c == 27)

break;

} // for (flag)

if (kbhit ())

getch ();

}

void test8255input (void)

{

unsigned long x, y, loop = 0L;

unsigned char c;

outportb (ioport + 3, 0x9b);

outportb (ioport + 7, 0x9b);

for (loop = 0;;loop++) {

x = inportb (ioport + 2);

x = (x << 8) | inportb (ioport + 1);

x = (x << 8) | inportb (ioport);

y = inportb (ioport + 6);

y = (y << 8) | inportb (ioport + 5);

y = (y << 8) | inportb (ioport + 4);

gotoxy (3, 16);

cprintf("Loop: %8ld : ", loop);

dumpbits (y);

dumpbits (x);

if (kbhit ()) {

c = getch ();

if (c == 27)

break;

} // for (i)

} // for (flag)

if (kbhit ())

getch ();

}

void test8253 (void)

{

unsigned long loop = 0L;

unsigned duration = 4;

int step = 0;

int i, j;

unsigned char c;

outportb (ioport + 3, 0x9b); //All input

outportb (ioport + 7, 0x89); //Port A, B output and Port C input

outportb (ioport + 11, 0x36); //Counter 0: Mode 3, LSB + MSB

outportb (ioport + 8, 10);

outportb (ioport + 8, 0);

outportb (ioport + 11, 0x76); //Counter 1: Mode 3, LSB + MSB

outportb (ioport + 9, 10);

outportb (ioport + 9, 0);

outportb (ioport + 11, 0xb6); //Counter 2: Mode 3, LSB + MSB

outportb (ioport + 10, 10);

outportb (ioport + 10, 0);

//  outportb (ioport + 5, 0x2a);

gotoxy (3, 17);

cprintf ("Please use probe to check if OUT0, OUT1, and OUT2 are blinking.");

for (loop = 0;;loop++) {

for (i = 0; i < 10; i++) {

outportb (ioport + 5, 0x2a);

for (j = 0; j <10; j++);

outportb (ioport + 5, 0x3f);

for (j = 0; j <10; j++);

}

gotoxy (3, 16);

cprintf("Loop: %8ld : ", loop);

delay (duration);

if (kbhit () || step) {

c = getch ();

if (c == 27)

break;

else if (c == ' ')

step = !step;

else if (c == '+') {

if (duration > 1)

duration >>= 1;

}

else if (c == '-') {

if (duration < 512)

duration <<= 1;

}

else if (!step)

getch ();

} //if

if (c == 27)

break;

} // for (flag)

if (kbhit ())

getch ();

}

void main()

{

int r;

int flag = 1;

char ch;

ioport = 0x1b0;

while (flag) {

clrscr();

cputs ("\n\n\n\r");

cputs ("                     8255/8253 I/O CARD Q.C. Program V1.2\n\r");

cputs ("                     ====================================\n\r");

cputs ("\n\r");

cprintf ("                     I/O Address: %04X\n\r", ioport);

cputs ("\n\r");

cputs ("                     (0): Change I/O setting\n\r");

cputs ("                     (1): 8255 Loopback test\n\r");

cputs ("                     (2): Output Test\n\r");

cputs ("                     (3): Input Test\n\r");

cputs ("                     (4): Counter Test (8253)\n\r");

cputs ("                     ESC: EXIT\n\r");

cputs ("                     Choose:");

do {

ch = getch();

} while (ch != '0' && ch != '1' && ch != '2' && ch != '3' && ch != '4' && ch != 27);

switch (ch) {

case 27:

flag = 0;

break;

case '0':

if (ioport == 0x1b0)

ioport = 0x300;

else if (ioport == 0x300)

ioport = 0x360;

else if (ioport == 0x360)

ioport = 0x1b8;

else

ioport = 0x1b0;

break;

case '1':

test8255loopback ();

break;

case '2':

test8255output ();

break;

case '3':

test8255input ();

break;

case '4':

test8253 ();

break;

}

}

} /* end of main */